| A. Building Air Leakage – General Information |  |  |
| --- | --- | --- |
| 01 | Test Procedure Used |  |
| 02 | Building Air Leakage Target from CF1R |  |
| 03 | Indoor Temperature During Test (°F) |  |
| 04 | Outdoor Temperature During Test (°F) |  |
| 05 | Blower Door Location |  |
| 06 | Building Elevation (ft) |  |
| 07 | Building Volume (ft3) |  |
| 08 | Date of the Diagnostic Test for this Dwelling |  |

| B. Diagnostic Equipment Information |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 01 | Number of Manometers Used to Measure Home Pressurization |  |  |  |  |  |  |  |
| 02 |  | 03 |  | 04 |  | 05 |  | 06 |
| Manometer Make |  | Manometer Model |  | Manometer Serial Number |  | Manometer Calibration Date |  | Manometer Calibration Status |
| 07 | Number of Fans Used to Pressurize Home |  |  |  |  |  |  |  |
| 08 |  |  | 09 |  | 10 |  | 11 |  |
| Fan Make |  |  | Fan Model |  | Fan Serial Number |  | Fan Configuration (rings) |  |

ENV20c – Multi-Point Air Tightness Test

| C. Envelope Leakage Diagnostic Test |  |  |
| --- | --- | --- |
| 01 | Name and Version of ASTM E779-10 Compliant Software used for Multi-Point Test |  |
| 02 | Pre-Test Baseline Building Pressure |  |
| 03 | Time Average Period of Meter |  |
| 04 | Test Methodology |  |
| 05 | Unadjusted Building Pressure Target |  |
| 06 | Unadjusted Building Pressure Measured |  |
| 07 | Induced Building Pressure |  |
| 08 | A minimum of eight readings were taken spaced evenly between 15 Pa and 60 Pa (or highest attainable pressure). |  |
| 09 | Post-Test Baseline Building Pressure |  |
| 10 | Corrected CFM50 (from software) |  |

D. Altitude and Temperature Correction (not used, performed by blower door software)

| E. Accuracy Adjustment |  |  |
| --- | --- | --- |
| 01 | Percent Uncertainty @ 95% Confidence Level (from software) |  |
| 02 | Accuracy Level |  |
| 03 | Accuracy Adjustment Factor |  |
| 04 | Adjusted CFM50 (measured air leakage rate) |  |

| F. Compliance Statement |  |
| --- | --- |
| 01 |  |

| G. Additional Requirements for Compliance |  |
| --- | --- |
| 01 | Open all interior doors and access including those to closets and those between a conditioned basement and attic. |
| 02 | HVAC supply and return register dampers shall be fully open. |
| 03 | Temporarily sealing of combustion flues and intermittent exhaust fans are not allowed. Some examples are: combustion flues, fresh air intakes, dryer vents, bathroom and kitchen exhaust vents and fire place. |
| 04 | Continuously operated ventilation devices like energy recovery ventilators may be sealed. |
| 05 | Multifamily – Each dwelling unit must be tested individually and shown to meet the leakage requirements. Pressurization of the adjacent dwelling units while conducting this test is not allowed. |
| The responsible person’s signature on this compliance document affirms that all applicable requirements in this table have been met. |  |

| Documentation Author's Declaration Statement |  |  |  |
| --- | --- | --- | --- |
| - I certify that this Certificate of Installation documentation is accurate and complete. |  |  |  |
| Documentation Author Name: |  | Documentation Author Signature: |  |
| Documentation Author Company Name: |  | Date Signed: |  |
| Address: |  | CEA/HERS Certification Identification (If applicable): |  |
| City/State/Zip: |  | Phone: |  |
| Responsible Person's Declaration statement |  |  |  |
| ### I certify the following under penalty of perjury, under the laws of the State of California: ### The information provided on this Certificate of Installation is true and correct. ### I am either: a) a responsible person eligible under Division 3 of the Business and Professions Code in the applicable classification to accept responsibility for the system design, construction, or installation of features, materials, components, or manufactured devices for the scope of work identified on this Certificate of Installation and attest to the declarations in this statement, or b) I am an authorized representative of the responsible person and attest to the declarations in this statement on the responsible person’s behalf. - The constructed or installed features, materials, components or manufactured devices (the installation) identified on this Certificate of Installation conforms to all applicable codes and regulations and the installation conforms to the requirements given on the Certificate of Compliance, plans, and specifications approved by the enforcement agency. - I understand that a HERS rater will check the installation to verify compliance and if such checking determines the installation fails to comply, I am required to offer any necessary corrective action at no charge to the building owner. - I will ensure that a registered copy of this Certificate of Installation shall be posted, or made available with the building permit(s) issued for the building, and made available to the enforcement agency for all applicable inspections. I understand that a registered copy of this Certificate of Installation is required to be included with the documentation the builder provides to the building owner at occupancy. |  |  |  |
| Responsible Builder/Installer Name: | Responsible Builder/Installer Signature: |  |  |
| Company Name: (Installing Subcontractor or General Contractor or Builder/Owner) | Position With Company (Title): |  |  |
| Address: | CSLB License: |  |  |
| City/State/Zip: | Phone: |  | Date Signed: |
| Third Party Quality Control Program (TPQCP) Status: | Name of TPQCP (if applicable): |  |  |

CF2R-ENV-20c-H User Instructions

Section A. Building Air Leakage – General Information

- Select the appropriate test procedure. This selection will determine which version of this document will be used (a, b, c, d, or e) and therefore which data must be collected. Note that single-point tests can only be used under certain conditions. Note that newer manometers have automatic functions for compensating for baseline (automatic baseline) and compensating for house pressures other than the target (@50 Pa). It is preferable to use these, when available, however if these automatic functions are to be used, they must be used for BOTH automatic baseline and pressure compensation.

- This number is automatically pulled from the CF1R and is the target maximum that was entered by the documentation author. If this number cannot be achieved, the performance compliance calculations can be redone with a higher number or without the requirement for building air leakage.

- Enter the indoor temperature measured at the time that the building air leakage test was performed.

- Enter the outdoor temperature measured at the time that the building air leakage test was performed.

- Provide a brief description of the location where the blower door was installed for the test. Examples: “front entry door on west side of house”, “door between house and garage”, “large window in family room”.

- Enter the building elevation; use the value for the closest city found in Joint Appendix JA2.2. Only elevations higher than 5,000 feet require an adjustment to the calculations.

- This number is automatically pulled from the CF1R. It is used to calculate air changes.

- Enter the date that the building leakage test data was collected.

Section B. Diagnostic Equipment Information

- Enter the number of manometers used to measure the home pressurization. If more than one system is used, the fan flow numbers need to be manually added together, unless blower door software is used that will accommodate multiple fan systems running simultaneously.

- Enter the make (brand) of the manometer used to collect the building air leakage data. Examples: Retrotec, Energy Conservatory.

- Enter the model of the manometer used to collect the building air leakage data. Examples: DM-2 Mark II, DG700.

- Enter the serial number of the manometer used to collect the building air leakage data.

- Enter the most recent date that the manometer was calibrated by following manufacturer’s calibration specifications.

- This field is automatically filled. If the calibration date was more than 12 months prior to the test date entered in Row A.8 above, an error will appear.

- Enter the number of blower door fan systems required to run simultaneously to pressurize the home for the building air leakage test. If more than one system is used, the fan flow numbers need to be manually added together, unless blower door software is used that will accommodate multiple fan systems running simultaneously.

- Enter the make (brand) of the fan used to collect the building air leakage data. Examples: Retrotec, Energy Conservatory.

- Enter the model of the fan used to collect the building air leakage data. Examples: US1000, Q46, BD3, BD4.

- Enter the serial number of the fan used to collect the building air leakage data.

- Enter the fan configuration shown on the meter. This is sometimes referred to as “range configuration”, “CONFIG” or “rings”. Examples: Open, A, B, C8.

Section C. Envelope Leakage Test (ENV20c)

- This version of the MCH-24 requires the use of an ASTM E779-10 compliant software, typically provided by the blower door manufacturer. Confirm with the software vendor that it is compliant. Enter the name and version here.

- Enter the Pre-Test Baseline Building Pressure.

- Enter the Time Average Period used on the manometer during the test. Must be at least 10 seconds.

- Select the type of test being performed: Pressurization (air blowing into house) or Depressurization (air blowing out of house). All blower door induced pressures are to be negative relative to outside.

- This field is automatically calculated. The Unadjusted Building Pressure Target is the Pre-test Baseline Building Pressure plus the target building pressure (-60 Pa).

- Enter the Measured Unadjusted Building Pressure straight from the manometer. It should be as close to the Unadjusted Building Pressure Target as possible. All blower door induced pressures for the depressurization tests are to be negative relative to outside.

- Enter the Induced Building Pressure straight from the manometer. All blower door induced pressures for the depressurization tests are to be negative relative to outside.

- When using the software for a multi-point test, a minimum of eight measurements must be taken over a range of pressures. This is where the user acknowledges that this was done.

- Enter the Post Test Baseline Building Pressure from the manometer.

- Enter the final Corrected CFM50 reading from the software.

Section D. Altitude and Temperature Correction (not used)

Section E. Accuracy Adjustment

- The software will provide a “Percent Uncertainty” value based on the readings taken. Enter that value here

- This field is automatically calculated. If the Percent Uncertainty level is 10% or less, the Accuracy Level is “Standard”. If the Percent Uncertainty level is greater than 10%, the Accuracy Level is “Reduced”.

- This field is automatically calculated:

  - If the Accuracy Level is “Standard”, the Accuracy Adjustment Factor will be 1 (no adjustment)

  - If the Accuracy Level is “Reduced”, the Accuracy Adjustment Factor will be adjusted by the Percent Uncertainty.

- This field is automatically calculated. The Adjusted CFM50 is the Corrected CFM50 multiplied by the Accuracy Adjustment Factor.

Section F. Compliance Statement

- This field is automatically calculated. A check is performed to make sure that the meter has been properly calibrated and that the measured infiltration is less than the target infiltration.

Section G. Additional Requirements for Compliance

- This statement must be true (or not applicable) for the test to conform to the protocols.

- This statement must be true (or not applicable) for the test to conform to the protocols.

- This statement must be true (or not applicable) for the test to conform to the protocols.

- This statement must be true (or not applicable) for the test to conform to the protocols.

- This statement must be true (or not applicable) for the test to conform to the protocols.

| A. Building Air Leakage – General Information |  |  |
| --- | --- | --- |
| 01 | Test Procedure Used | <<user input, pull down list: Single-Point Test with Manual Meter – Display ENV-20a below; Single-Point Test with Automatic Meter – Display ENV-20b below; Multi-Point Test – Display ENV-20c below; Repeated Single Point with Manual Meter – Display ENV-20d below; Repeated Single Point with Automatic Meter – Display ENV-20e below>> |
| 02 | Building Air Leakage Target from CF1R | <<number pulled from CF1R>> |
| 03 | Indoor Temperature During Test (°F) | <<user input, degF>> |
| 04 | Outdoor Temperature During Test (°F) | <<user input, degF>> |
| 05 | Blower Door Location | <<user input, text, maximum 50 characters>> |
| 06 | Building Elevation (ft) | <<user input, Ft>> |
| 07 | Building Volume (ft3) | <<pull from CF1R>> |
| 08 | Date of the Diagnostic Test for this Dwelling | <<user input: date (use date format validation control)>> |

| B. Diagnostic Equipment Information |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 01 | Number of Manometers Used to Measure Home Pressurization |  |  |  | <<user input, integer>> For entries >1, duplicate lines B. 2-6 |  |  |  |
| 02 |  | 03 |  | 04 |  | 05 |  | 06 |
| Manometer Make |  | Manometer Model |  | Manometer Serial Number |  | Manometer Calibration Date |  | Manometer Calibration Status |
| <<user input, text, maximum 50 characters>> |  | <<user input, text, maximum 50 characters>> |  | <<user input, text, maximum 50 characters>> |  | <<user input, text (Date), maximum 50 characters>> |  | <<calculated field: if manometer Calibration Date in B. 5 is within 12 months of the date of the diagnostic test A. 8, then display message: Manometer Calibration is valid"; else display message: "WARNING - Manometer Calibration is expired. A manometer with current calibration is required in order to comply with this Building Leakage Diagnostic test">> |
| 07 | Number of Fans Used to Pressurize Home |  |  |  | <<user input, integer>> For entries >1, duplicate lines B. 8-11 |  |  |  |
| 08 |  |  | 09 |  | 10 |  | 11 |  |
| Fan Make |  |  | Fan Model |  | Fan Serial Number |  | Fan Configuration (rings) |  |
| <<user input, text, maximum 50 characters>> |  |  | <<user input, text, maximum 50 characters>> |  | <<user input, text, maximum 50 characters>> |  | <<user input, text, maximum 50 characters>> |  |

ENV20c – Multi-Point Air Tightness Test

| C. Envelope Leakage Diagnostic Test |  |  |
| --- | --- | --- |
| 01 | Name and Version of ASTM E779-10 Compliant Software used for Multi-Point Test | <<user input, text, maximum 50 characters>> |
| 02 | Pre-Test Baseline Building Pressure | <<user input (Pa)>> (Resolution of 0.1 Pa) |
| 03 | Time Average Period of Meter | <<user entry but must be no less than 10 (seconds)>> |
| 04 | Test Methodology | <<user input, pull down list: Pressurization; Depressurization>> |
| 05 | Unadjusted Building Pressure Target | <<calculated value: if C. 4 equal to Pressurization use equation, 60 + row C. 2; if C. 4 equal to Depressurization use equation, -60 + row C . 2>> (Resolution of 0.1 Pa) |
| 06 | Unadjusted Building Pressure Measured | <<user entered value>> (Resolution of 0.1 Pa) |
| 07 | Induced Building Pressure | <<calculated value, row C. 6 – row C. 2>> |
| 08 | A minimum of eight readings were taken spaced evenly between 15 Pa and 60 Pa (or highest attainable pressure). | <<user to acknowledge this requirement to continue>> |
| 09 | Post-Test Baseline Building Pressure | <<user input (Pa)>> (Resolution of 0.1 Pa) |
| 10 | Corrected CFM50 (from software) | << user entry of value calculated by software >> (Resolution of 1 CFM) |

D. Altitude and Temperature Correction (not used, performed by blower door software)

| E. Accuracy Adjustment |  |  |
| --- | --- | --- |
| 01 | Percent Uncertainty @ 95% Confidence Level (from software) | << user entry of value calculated by software>> |
| 02 | Accuracy Level | <<calculated, if row E. 1 ≤ 10, “Standard”; if row E. 1 > 10, “Reduced”>> |
| 03 | Accuracy Adjustment Factor | <<calculated, if row E. 2 is “Standard”=1; If it is “Reduced”, 1+ (E.1 % uncertainty/100)>> |
| 04 | Adjusted CFM50 (measured air leakage rate) | <<calculated, row C. 10 * row E. 3>> |

| F. Compliance Statement |  |
| --- | --- |
| 01 | << if manometer Calibration Date in B. 5 is within 12 months of the date of the diagnostic test A. 8 and if Adjusted CFM50 Leakage in E. 4 is less than or equal to the Building Air Leakage Rate Target in A. 2 then display text: "Building Passes Envelope Leakage Test"; if manometer Calibration Date in B. 5 is more than 12 months from the date of the diagnostic test A. 8 or if Adjusted CFM50 Leakage in E. 4 is more than the Building Air Leakage Rate Target in A. 2 then display text: "Building Fails Envelope Leakage Test">> |

| G. Additional Requirements for Compliance |  |
| --- | --- |
| 01 | Open all interior doors and access including those to closets and those between a conditioned basement and attic. |
| 02 | HVAC supply and return register dampers shall be fully open. |
| 03 | Temporarily sealing of combustion flues and intermittent exhaust fans are not allowed. Some examples are: combustion flues, fresh air intakes, dryer vents, bathroom and kitchen exhaust vents and fire place. |
| 04 | Continuously operated ventilation devices like energy recovery ventilators may be sealed. |
| 05 | Multifamily – Each dwelling unit must be tested individually and shown to meet the leakage requirements. Pressurization of the adjacent dwelling units while conducting this test is not allowed. |
| The responsible person’s signature on this compliance document affirms that all applicable requirements in this table have been met. |  |