| A. General Information |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 01 | Project Name: |  | 02 | Date Prepared: |  |
| 03 | Project Location: |  | 04 | Building Front Orientation (deg or cardinal): |  |
| 05 | CA City: |  | 06 | Number of Dwelling Units: |  |
| 07 | Zip Code: |  | 08 | Fuel Type: |  |
| 09 | Climate Zone: |  | 10 | Total Conditioned Floor Area (ft2): |  |
| 11 | Building Type: |  | 12 | Slab Area (ft2): |  |
| 13 | Project Scope: |  | 14 | Exceptions to Fenestration U-factor & SHGC 150.1(c)3A: |  |

| B. Opaque Surface Details – Framed (Section 150.1(c)1) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |  | 07 | 08 | 09 | 10 | 11 |
| Tag/ID | Assembly Type | Frame Type | Frame Depth (inches) | Frame Spacing (inches) | Proposed |  |  |  |  | Required | Comments |
|  |  |  |  |  | Cavity R-value | Continuous Insulation R-value | U-Factor | Appendix JA4 Reference |  | U-Factor from Package A |  |
|  |  |  |  |  |  |  |  | Table | Cell |  |  |

| C. Opaque Surface Details – Non-framed (Section 150.1(c)1) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 | 07 | 08 | 09 | 10 | 11 |
| Tag/ID | Assembly Type | Assembly Materials | Thickness (inches) | Proposed |  |  |  |  | Required | Comments |
|  |  |  |  | Core Insulation R-value | Continuous Insulation R-value | U-Factor | Appendix JA4 Reference |  | U-Factor from Package A |  |
|  |  |  |  |  |  |  | Table | Cell |  |  |

| D. Opaque Surface Details – Mass Walls (Section 150.1(c)1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 | 07 |  | 08 |  | 09 | 10 | 11 |  | 12 |  |
| Tag/ID | Walls Above Grade | Mass Type | Mass Thickness (inches) | Interior Furring Strip Thickness (inches) | Exterior Furring Strip Thickness (inches) | Proposed |  |  |  |  |  | Required |  |  |  |
|  |  |  |  |  |  | Interior Insulation |  | Exterior Insulation |  | Appendix JA4 Reference |  | Interior Insulation |  | Exterior Insulation |  |
|  |  |  |  |  |  | R-value | U-factor | R-value | U-factor | Table | Cell | R-value | U-factor | R-value | U-factor |

| E. Slab Insulation (Table 150.1-A) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 |  | 03 |  | 04 |
| Floor Type | Proposed |  | Required |  | Comments |
|  | Insulation R-value | Insulation U-factor | Insulation R-value | Insulation U-factor |  |
| Note: - Heated slab floors require mandatory slab insulation (see Table 110.8-A). |  |  |  |  |  |

F. Ceiling/Roof Insulation (Section 150.1(c)1A)

| 01 | 02 | 03 | 04 | 05 | 06 | 07 |
| --- | --- | --- | --- | --- | --- | --- |
| Option | Air Space (Yes/No) | Above Roof Deck R-value | Below Roof Deck R-value | Ceiling Insul. R-value | Radiant Barrier Required? | Vented Attic? (Yes/No) |
| Note: - Where insulation is installed above the roofing membrane, or above the layer used to seal the roof from water penetration, the insulation shall have a maximum water absorption of 0.3 percent by volume when tested according to ASTM Standard C272. |  |  |  |  |  |  |

| G. Roofing Products (Cool Roof) (Section 150.1(c)11) |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 | 07 | 08 | 09 | 10 | 11 | 12 | 13 |
| Tag/ID | Mass Roof 25 lb/ft2 or greater | Roof Pitch | Method of Compliance | Product Type | CRRC Product ID Number | Proposed |  |  |  | Minimum Required |  |  |
|  |  |  |  |  |  | Initial Solar Reflectance | Aged Solar Reflectance | Thermal Emittance | SRI (Optional) | Aged Solar Reflectance | Thermal Emittance | SRI (Optional) |
| Notes: - Any roof area covered by building integrated photovoltaic panels and solar thermal panels is exempt from the above Cool Roof requirements. - Liquid field applied coatings must comply with installation criteria from section 110.8(i)4. |  |  |  |  |  |  |  |  |  |  |  |  |

| H. Fenestration/Glazing Allowed Areas and Efficiencies (Section 150.1(c)3) |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 | 07 |
| Maximum Allowed Fenestration Area for All Orientations (ft2) | Maximum Allowed West-Facing Fenestration Area Only (ft2) | Maximum Allowed U-factor (Windows) | Maximum Allowed U-factor (Skylights) | Maximum Allowed SHGC (Windows) | Maximum Allowed SHGC (Skylights) | Comments |

| I. Fenestration Proposed Areas and Efficiencies Note: If meeting Exception 1 to 150.1(c)3A, Installing ≤ 3ft2 glass in door, it is assumed to meet the minimum required U-factor (0.32) & SHGC (0.25). If meeting Exception 1 to 150.1(c)3A, Installing ≤ 3ft2 tubular skylight, it is assumed to meet the minimum required U-factor (0.55) & SHGC (0.30). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 |  | 04 | 05 | 06 | 07 | 08 | 09 | 10 | 11 | 12 | 13 |  | 14 |
| Tag/ID | Fenestration Type | Frame Type |  | Dynamic Glazing | Orientation N, S, W, E | Number of Panes | Proposed Fenestration Area (ft2) | Proposed West Facing Fenestration Area (ft2) | Proposed U-factor | Source | Proposed SHGC | Source | Exterior Shading Device |  | Combined SHGC from CF1R-ENV-03 |
| 15 | Total Proposed Fenestration Area |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 16 | Maximum Allowed Fenestration Area |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 17 | Compliance Statement: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 18 | Total Proposed West-Facing Fenestration Area |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 19 | Maximum Allowed West-Facing Fenestration Area |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 20 | Compliance Statement: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 21 | Proposed Fenestration U-factor (Windows) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 22 | Required Fenestration U-factor (Windows) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 23 | Compliance Statement: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 24 | Proposed Fenestration SHGC (Windows) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 25 | Required Fenestration SHGC (Windows) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 26 | Compliance Statement: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 27 | Proposed Fenestration U-factor (Skylights) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 28 | Required Fenestration U-factor (Skylights) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 29 | Compliance Statement: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 30 | Proposed Fenestration SHGC (Skylights) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 31 | Required Fenestration SHGC (Skylights) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 32 | Compliance Statement: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| J. Space Conditioning (SC) Systems – Heating/Cooling/Ducts (Section 150.1(c)7) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 | 07 | 08 | 09 | 10 | 11 | 12 |
| SC System Identification or Name | Heating System Type | Heating Efficiency Type | Heating Efficiency Value | Cooling System Type | Cooling Efficiency SEER | Cooling Efficiency EER | Distribution System Type | Duct Location | Duct R-value | Thermostat Type | Comments |
| Notes: - Central gas furnaces have a minimum efficiency of 81% AFUE, heat pumps 8.0 HSPF. Any gas heating appliance sold in California will meet the minimum appliance efficiency standard and is allowed. Heat pumps and mini-split heat pumps are the only type of electric heating system allowed. - Central cooling systems and heat pumps have a minimum efficiency of 14 SEER. Any cooling appliance sold in California will meet the minimum appliance efficiency standard and is allowed. - The prescriptive requirements preclude the use of bypass ducts in association with zonally controlled systems. A HERS Rater shall verify that zonally controlled systems have no bypass ducts. |  |  |  |  |  |  |  |  |  |  |  |

| K. Ventilation Cooling in Climate Zones 8-14 (Section 150.1(c)12) |  |
| --- | --- |
| 01 | 02 |
| Required Airflow Rate (CFM) (1.5 CFM per ft2 of Conditioned Floor Area) | Minimum Attic Vent Free Area (in2) (Required Airflow Rate x 0.192) |

| L. Water Heating Systems (Section 150.1(c)8) List water heaters and boilers for both domestic hot water (DHW) heaters and hydronic space heating. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 | 07 | 08 | 09 | 10 | 11 | 12 | 13 | 14 | 15 |
| Water Heating System ID or Name | Water Heating System Type | Water Heater Type | # of Water Heaters in System | Water Heater Storage Volume (gal) | Fuel Type | Rated Input Type | Rated Input Value | Heating Efficiency Type | Heating Efficiency Value | Standby Loss (%) | Exterior Insul. R-Value | Back-Up Solar Savings Fraction | Central DHW System Distribution Type | Dwelling Unit DHW System Distribution Type |

| M. Space Conditioning Systems and Water Heating Systems in Multifamily Dwelling Units |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
| Dwelling Unit Name | Dwelling Unit Total CFA (ft2) | Central Water Heating System Identification or Name | Dwelling Unit Water Heating System Identification or Name | Dwelling Unit Space Conditioning System Identification or Name | Comments |

N. HERS Verification Summary The enforcement agency shall pay special attention to the HERS Measures specified in this checklist below. A registered Certificate of Verification for all the measures specified shall be submitted to the building inspector before final inspection.

Duct Leakage Verification- Section 150.0(m)11 - Duct leakage testing is required (Residential Appendix RA3.1) in all climate zones for ducted heating and cooling systems.

Zonally Controlled Systems – Bypass Dampers - Section 150.1(c)13 - If system is zonally controlled, no bypass ducts are allowed, as confirmed by HERS verification (See RCM Appendix F)

Refrigerant Charge Verification – Section 150.1(c)7a - Refrigerant Charge Testing is required (Residential Appendix RA3.2) in climate zones 2 and 8-15 for all air source A/C and heat pumps. - Some exceptions apply to factory charged package systems

Central System Air Handlers – Air Flow and Fan Efficacy Verification - Section 150.0(m)13 - Airflow (min 350 cfm/ton) and Fan Efficacy (max 0.58 Watts/cfm) on systems with ducted air conditioning will be field verified by a HERS rater or Return Duct and Filter System Design according to tables 150.0-B/C will be HERS verified - Heat-only systems with Central Fan Integrated (CFI) ventilation are required to have less than 0.58 Watts per cfm as verified by a HERS rater.

Indoor Air Quality Mechanical Ventilation - Mechanical ventilation airflow rate according to ASHRAE 62.2 is required to be verified by a HERS rater (RA3.7)

Quality Insulation Installation - Section 150.1(c)8ii - QII is required when installing a single gas or propane water heater with an input of 105,000 Btu per hour or less, rated volume less than or equal to 55 gallons and that meets the requirements of Sections 110.1 and 110.3. - The dwelling unit shall meet all of the requirements of Quality Insulation Installation (QII) as specified in Reference Appendix RA3.5 and be verified by a HERS rater.

| Documentation Author's Declaration Statement |  |
| --- | --- |
| - I certify that this Certificate of Compliance documentation is accurate and complete. |  |
| Documentation Author Name: | Documentation Author Signature: |
| Company: | Signature Date: |
| Address: | CEA/ HERS Certification Identification (if applicable): |
| City/State/Zip: | Phone: |
| Responsible Person's Declaration statement |  |
| ### I certify the following under penalty of perjury, under the laws of the State of California: - The information provided on this Certificate of Compliance is true and correct. - I am eligible under Division 3 of the Business and Professions Code to accept responsibility for the building design or system design identified on this Certificate of Compliance (responsible designer). - That the energy features and performance specifications, materials, components, and manufactured devices for the building design or system design identified on this Certificate of Compliance conform to the requirements of Title 24, Part 1 and Part 6 of the California Code of Regulations. - The building design features or system design features identified on this Certificate of Compliance are consistent with the information provided on other applicable compliance documents, worksheets, calculations, plans and specifications submitted to the enforcement agency for approval with this building permit application. - I will ensure that a registered copy of this Certificate of Compliance shall be made available with the building permit(s) issued for the building, and made available to the enforcement agency for all applicable inspections. I understand that a registered copy of this Certificate of Compliance is required to be included with the documentation the builder provides to the building owner at occupancy. |  |
| Responsible Designer Name: | Responsible Designer Signature: |
| Company: | Date Signed: |
| Address: | License: |
| City/State/Zip: | Phone: |

For assistance or questions regarding the Energy Standards, contact the Energy Hotline at: 1-800-772-3300

CF1R-NCB-01-E User Instructions

Minimum requirements for prescriptive compliance can be found in Building Energy Efficiency Standards Section 150.1(c), and Table 150.1-A (Package A). Completing these compliance documents will require that you have the Reference Appendices for the 2016 Building Energy Efficiency Standards, which contains the Joint Appendices used to determine climate zone and to complete the table for opaque surfaces. When the term CF1R is used it means the CF1R-NCB-01. Worksheets are identified by their entire name and subsequently by only the worksheet number, such as CF1R-ENV-02.

Instructions for tables with column numbers and row letters are given separately.

A. General Information

- Project Name: Identifying information, such as owner’s name.

- Date: Date of document preparation.

- Project Location: Legal street address of property or other applicable location identifying information.

- Building Front Orientation: Building front expressed in degrees, where North = 0, East = 90, South = 180, and West = 270. Indicate cardinal if it is a subdivision or multi-family project that will be built in multiple orientations. The Standards (Section 100.1) include the following additional details for determining orientation:

- Cardinal covers all orientations (for buildings that will be built in multiple orientations);

- North is oriented to within 45 degrees of true north, including 45 degrees east of north;

- East is oriented to within 45 degrees of true east, including 45 degrees south of east;

- South is oriented to within 45 degrees of true south, including 45 degrees west of south;

- West is oriented to within 45 degrees of true west, including 45 degrees south of west.

- CA City: Legal city/town of property.

- Number of Dwelling Units: 1 for single family, 2 or more for multi-family.

- Zip Code: 5-digit zip code for the project location (used to determine climate zone).

- Fuel Type: Natural Gas, or Liquefied Propane Gas.

- Climate zone: From Joint Appendix JA2.1.1.

- Total Conditioned Floor Area: Enter the total new conditioned floor area in ft2, as measured from the outside of exterior walls. If the project is an addition, this form is used only for additions that are greater than 1,000 ft2.

- Building Type: Single Family (includes duplex), or Multi-Family (a building that shares common walls and common floors or ceilings).

- Slab Area: Area of the first floor slab (if any) in ft2.

- Project Scope: Newly constructed building, or new addition greater than 1,000 ft2.

- Exceptions to Fenestration U-factor and SHGC: Installing less than or equal to 3 ft2 glass in door, Installing less than or equal to 3 ft2 tubular skylight, Installing less than or equal to 16 ft2 skylight, or Not Applicable.

B. Opaque Surface Details – Framed

- Tag/ID: A label (if any) from the plans, such as A1.4 or wall.

- Assembly Type: Roof, Ceiling, Wall, Floor.

- Frame type: Wood or Metal.

- Frame Depth: Nominal dimensions (in inches) of framing material; such as 2x4 or 2x6.

- Frame Spacing: 16, 24, or 48 (inches on center).

- Proposed Cavity R-value: Cavity R-value of insulation installed between framing members.

Proposed Continuous Insulation R-value: R-value of rigid or continuous insulation (not interrupted by framing).

- Proposed U-factor: The U-factor for the proposed assembly. Must be less than or equal to Column 10 or have an attached CF1R-ENV-02-E to show that a weighted U-factor for multiple assemblies will meet the maximum value in Column 10.

- Appendix JA4 Table: Table number used to determine the R-value or U-factor (e.g., an ICF wall is 4.3.13).

- Appendix JA4 Cell: Cell number used to determine the R-value or U-factor (e.g., an 8-inch thick ICF wall with 2 inches of EPS (R-15.4) is A6).

- Required U-factor from Package A: Value required based on climate zone and assembly type.

- Comments: Any notes regarding location, unique conditions, or attachments.

C. Opaque Surface Details – Non-framed

- Tag/ID: A label (if any) from the plans, for example, A1.4 or wall.

- Assembly Type: Roof, Wall.

- Assembly materials: SIP OSB, SIP I-Joist, SIP Single 2x, SIP Double 2x, see JA4 for guidance.

- Thickness: Thickness in inches.

- Proposed Core Insulation R-value: Insulation installed within the materials or on the inside. See Joint Appendix JA4 for guidance.

- Proposed Continuous Insulation R-value: Insulation installed on the exterior. See Joint Appendix JA4 for guidance.

- Proposed U-factor: Assembly U-factor from JA4 or CF1R-ENV-02. Must be less than or equal to Column 10.

- Appendix JA4 Table: Table number used to determine the R-value or U-factor (e.g., an ICF wall is 4.3.13).

- Appendix JA4 Cell: Cell number used to determine the R-value or U-factor (e.g., an 8-inch thick ICF wall with 2 inches of EPS (R-15.4) is A6).

- Required U-factor from Package A: Based on assembly type and climate zone.

- Comments: Any notes regarding location, unique conditions, or attachments.

D. Opaque Surface Details – Mass Walls

- Tag/ID: A label (if any) from the plans, for example, A1.4 or wall.

- Walls Above Grade: Yes or No.

- Mass Type: Clay Brick, Clay Hollow Unit, CMU Light Weight, CMU Medium Weight, CMU Normal Weight, Concrete, ICF. See JA4 for guidance.

- Mass Thickness: Thickness (in inches) of mass.

- Interior Furring Strip Thickness: If furring strips are required to meet the required wall R-value or U-factor shown in Columns 11 and 12, indicate the thickness of the furring strip (in inches). See Table 4.3.14 of Joint Appendix 4.

- Exterior Furring Strip Thickness: If furring strips are required to meet the required wall R-value or U-factor shown in Columns 11 and 12, indicate the thickness of the furring strip (in inches). See Table 4.3.14 of Joint Appendix 4.

- Proposed Interior Insulation R-value or U-factor: Enter either the R-value or U-factor of proposed insulation on the inside surface of the mass wall. See Column 10 for the required interior insulation value for the wall type selected. See JA4 for guidance. Use the same descriptor (R-value or U-factor) throughout Table D.

- Proposed Exterior Insulation R-value or U-factor: Enter either the R-value or U-factor of proposed insulation on the outside surface of the mass wall. See Column 11 for the required exterior insulation value for the wall type selected. See JA4 for guidance.

- Appendix JA4 Table: Table number used to determine the R-value or U-factor (e.g., an ICF wall is 4.3.13).

- Appendix JA4 Cell: Cell number used to determine the R-value or U-factor (e.g., an 8-inch thick ICF wall with 2 inches of EPS (R-15.4) is A6).

- Required Interior Insulation R-value or U-factor: The required R-value or U-factor (whichever descriptor was selected in Column 6) for interior insulation will be completed based on the Table 150.1-A requirements for the wall type.

- Required Exterior Insulation R-value or U-factor: The required R-value or U-factor (whichever descriptor was selected in Column 7) for exterior insulation will be completed based on the Table 150.1-A requirements for the wall type.

E. Slab Insulation

Slab edge performance specifications and installation criteria are found in Sections 150.0(l) and 150.1(c)1D (Table 150.1-A). Requirements vary by climate zone and slab conditions.

- Floor type: Types include slab-on-grade or raised slab.

- Slab-on-grade floors require slab edge insulation in climate zone 16 only.

- Raised slabs must be insulated to R8 in climate zones 1, 2, 11, 13, 14 and 16, R-4 in climate zones 12 and 15, and no insulation is required in climate zones 3-10.

- Proposed R-value: When required, insulation can be specified by either R-value or U-factor. If specifying an R-value complete column 2.

Proposed U-Factor: When required, specify the U-factor of the proposed insulation in Column 3.

- Required Insulation R-value: Whichever descriptor was used (R-value or U-factor) in Columns 2 & 3 will be used to specify the value required, which will vary by climate zone and type of slab. Values are from Table 150.1-A.

Required Insulation U-factor: Whichever descriptor was used (R-value or U-factor) in Columns 2 & 3 will be used to specify the value required, which will vary by climate zone and type of slab. Values are from Table 150.1-A.

- Comments: Any notes regarding location, unique conditions, or attachments.

NOTE:	A suggestion is provided to highlight that there is a mandatory slab edge insulation requirement for heated slab floors. Since mandatory requirements are not listed on the Certificate of Compliance, this is provided for information purposes only. The specific requirements are in Sections 110.8(g) and Table 110.8-A.

F. Roof/Ceiling Insulation

- Option: Option A (Continuous Insulation), Option B (Below Roof Deck Insulation), or Option C.

- Air Space: Yes or No.

- Above Roof Deck R-value: This field will be auto populated. The required above roof deck R-value will vary depending on climate zone and whether or not there is an air space.

- Below Roof Deck R-value: This field will be auto populated. The required below roof deck R-value will vary depending on climate zone and whether or not there is an air space.

- Insulation R-value: This field will be auto populated. The required insulation R-value will vary depending on climate zone.

- Radiant Barrier Required?: This field will be auto populated. The radiant barrier requirement will vary depending on compliance option used and climate zone.

- Vented Attic?: Yes or No.

Note: If attic is unvented, prescriptive compliance cannot be used. Compliance may be attempted using the performance approach.

G. Roofing Products (Cool Roof)

Roofing requirements are found in Sections 110.8(i) and 150.1(c)11. Depending on the climate zone and roof slope, a cool roof (defined as a minimum aged solar reflectance and thermal emittance, or a minimum SRI) may be required by Package A.

Exceptions include (1) low-slope roofs (pitch 2:12 or less) in climate zones 1-12, 14 and 16; (2) steep-slope roofs (pitch greater than 2:12) in climate zones 1-9 and 16; (3) roof constructions that have thermal mass over the roof membrane with at least 25 lb/ft2; and (4) any roof area covered by building integrated photovoltaic panels and solar thermal panels (the area of roof not covered by photovoltaic panels would still need to meet any applicable cool roof requirements).

- Tag/ID: A label (if any) from the plans, such as R1.

- Mass Roof 25 lb/ft2 or Greater: Yes or No. Mass roofs are not required to have a cool roof even if the climate zone specifies minimum performance requirements.

- Roof Pitch: Expressed as 4:12, for example, which means the roof rises 4 feet within a span of 12 feet. When roofs have multiple pitches the requirements are based on the pitch of 50% or more of the roof.

- Method of Compliance: Indicate if the method of compliance is going to be based on Aged Solar Reflectance and Thermal Emittance, or is it going to be based on the Solar Reflectance Index (SRI).

- Product Type: See Cool Roof Rating Council’s Directory. Generally product types include single-ply roof, wood shingles, asphalt roof, metal roof, tile roof.

- The CRRC Product ID Number is obtained from the Cool Roof Rating Council’s Rated Product Directory at www.coolroofs.org/products/results. Products are listed by manufacturer, brand, type of installation, roofing material, and color, as well as product performance.

- Proposed Initial Solar reflectance: Based on the product chosen from the Cool Roof Rating Council’s Rated Product Directory. If using default assumption indicate NA since the Aged Solar Reflectance is available.

- Proposed Aged Solar Reflectance: Value is from the Cool Roof Rating Council’s Rated Product Directory. If the aged value is not available, calculate the calculated Aged Solar Reflectance using the Solar Reflectance Index (SRI) Calculation worksheet located on the California Energy Commission website or the aging equation ρaged=[0.2+β[ρinitial-0.2], where ρinitial = the initial solar reflectance and soiling resistance β is listed by product type below.

VALUES OF SOILING RESISTANCE β BY PRODUCT TYPE

| Product Type | CRRC Product Category | β |
| --- | --- | --- |
| Field-Applied Coating | Field-Applied Coating | 0.65 |
| Other | Not A Field-Applied Coating | 0.70 |

- Proposed Thermal Emittance: From the product specification default value. If using a calculated SRI place the Thermal Emittance used to calculate SRI.

- Proposed SRI (optional): It is optional to meet the SRI but if chosen to do so, use the Solar Reflectance Index (SRI) Calculation Worksheet found on the California Energy Commission website http://energy.ca.gov/title24/2013standards/documents/solar_reflectance/.

- Minimum Required Aged Solar Reflectance: Based on climate zone and roof slope.

- Minimum Required Thermal Emittance: Based on climate zone and roof slope.

- Minimum Required SRI (optional): Based on climate zone and roof slope.

If the cool roofing requirements will be met by a liquid field applied coating, Section 110.8(i)4 requires the coating be applied across the entire roof surface and meet the dry mil thickness, or coverage recommended by the manufacturer.

H. Fenestration/Glazing Allowed Areas and Efficiencies

  - Maximum Allowed Fenestration Area for All Orientations: Calculated value based on conditioned floor area multiplied by 20% for all orientations.

- Maximum Allowed West-Facing Fenestration Area Only: Calculated value based on conditioned floor area multiplied by 5% (Used in climate zones 2, 4, and 6-16 for west-facing fenestration).

- Maximum Allowed U-factor (Windows): Maximum U-factor from Package A or Table 150.1-A. This field will always be 0.32.

- Maximum Allowed U-factor (Skylights): Maximum U-factor from Package A or Table 150.1-A. This field will almost always be 0.32 unless meeting one of the Exceptions to 150.1(c)3A. If meeting one of the Exceptions, this field will be 0.55.

- Maximum Allowed SHGC (Windows): Maximum SHGC from Package A or Table 150.1-A. This field will either be 0.25 or N/A, depending on the climate zone. N/A means there is no maximum SHGC required in this climate zone. The SHGC will be the area weighted averaged, CF1R-ENV-02, with other higher fenestration windows.

- Maximum Allowed SHGC (Skylights): Maximum SHGC from Package A or Table 150.1-A. This field will almost always be 0.25 unless meeting one of the Exceptions to 150.1(c)3A. If meeting one of the Exceptions, this field will be 0.30.

- Comments: Any notes regarding location, unique conditions, or attachments.

I. Fenestration Proposed Areas and Efficiencies

- Tag/ID: Provide a name or designator for each unique type of fenestration surface. This designator should be used consistently throughout the plan set (elevations, finish schedules, etc.) such as Window-1, Skylight-1, etc. to identify each surface. It should also be consistently used on the other forms in the compliance documentation.

- Fenestration Type: Indicate the type of fenestration construction (e.g., Fixed Window, Operable Window, Skylight, Tubular Skylight, or Glass in Door).

NOTE: 	Doors with glazing are counted in one of two ways. The entire door area of a door with 50% or more glazing is considered fenestration. A door with less than 50% glazing can be considered as all fenestration, or can be calculated as the actual glass area with a 2-inch (0.17 ft) frame all around.

- Frame Type: Indicate the frame type as either metal, metal thermal break, or nonmetal.

- Dynamic Glazing: Indicate whether the fenestration has an integrated shading device, chromogenic glazing, or none for no dynamic glazing. Chromogenic glazing shall be considered separately from other fenestration types.

- Orientation: Orientation can be North, East, South, West. If documentation is for a building that may be built in any direction, in a climate zone that limits west-facing fenestration, complete this section assuming the side of the building with the most fenestration faces west.

NOTE: 	West includes any vertical fenestration oriented to within 45 degrees of true west, excluding 45 degrees south of west; any skylights oriented west; and skylights facing any direction with a pitch of less than 1:12.

- Number of Panes: Indicate the number of panes for each Tag/ID; is it a single, double, or triple pane window?

- Proposed Fenestration Area (ft2): Indicate the area (in ft2) of each exterior fenestration type, excluding west-facing fenestration.

- Proposed West Facing Fenestration Area (ft2): In climate zones 2, 4, and 6-16, indicate the area (in ft2) of each exterior west-facing fenestration type separately.

NOTE: 	Skylights installed in a roof with a pitch less than 1:12 are considered to face west.

- Proposed U-factor: Enter

(a) the NFRC U-factor based on the proposed brand and type of fenestration using National Fenestration Rating Council (www.nfrc.org) certified values, or

(b) the default value from Table 110.6-A, or

(c) the NA6.2 alternate default U-factor (for non-rated site-built fenestration only), or

(d) the Area-weighted Average from CF1R-ENV-02.

If any products (other than the exceptions) have a higher U-factor than 0.32, first complete a form CF1R-ENV-02 to calculate the Area-Weighted Average U-factor, which must be 0.32 or less, and attach it to the CF1R-NCB-01.

NOTE: 	(1) For the exceptions – up to 3 ft2 of tubular skylights and up to 16 ft2 of skylight area, enter 0.55.

(2) For the exception – up to 3 ft2 of glass in door, enter 0.32.

(3) Dynamic glazing is a glazing system that changes its performance U-factor and SHGC based on the physical environment. Dynamic glazing includes chromogenic glazing or integrated shading systems (this does not include internally or externally mounted shading devices). If using dynamic glazing, use the lowest tested U-factor and SHGC in Columns 9 and 10.

- Source: NFRC, Tables 110.6-A and 110.6-B, Equations NA6-1 and NA6-2, or Area-Weighted Average Worksheet (CF1R-ENV-02). The source of the U-factor data for the fenestration product.

- Proposed SHGC: In climate zones 2, 4, and 6-16, enter the SHGC from

(a) NFRC-rated certification information; or

(b) default Table 110.6-B; or

(c) the NA6.3 alternate default SHGC (for non-rated site-built fenestration only);, or

(d) the Area-weighted Average from CF1R-ENV-02.

If any products (other than the exceptions) have a higher SHGC than required by Package-A, first complete a form CF1R-ENV-02 to calculate the Area-Weighted Average SHGC and attach it to the CF1R-NCB-01.

NOTE: 	(1) For the exceptions – up to 3 ft2 of tubular skylights and up to 16 ft2 of skylight area, enter 0.30.

(2) For the exception – up to 3 ft2 of glass in door, enter 0.25.

- Source: NFRC, Tables 110.6-A and 110.6-B, Equations NA6-1 and NA6-2, or Area-weighted Average Worksheet (CF1R-ENV-02). The source of the SHGC data for the fenestration product.

- Exterior Shading Device: If exterior shading devices are used to meet the SHGC requirement, indicate the type of device (from Table S-1 of CF1R-ENV-03-E Solar Heat Gain Coefficient Worksheet) and attach the CF1R-ENV-03-E.

NOTE:	An exterior shading device is not used for products with an NFRC rated U-factor and SHGC based on a factory integrated shading device.

Chromogenic glazing shall be considered separately from other fenestration.

- Combined SHGC from CF1R-ENV-03: If exterior shading devices are combined with the SHGC value of the fenestration to meet the prescriptive SHGC requirements (as indicated in column I. 13), indicate the SHGC calculated on form CF1R-ENV-03 and attach the form for each window with an exterior shading device.

15.–32. Automatically completed entries; no user input required.

J. Space Conditioning (SC) Systems – Heating/Cooling/Ducts

- Space Conditioning System Identification or Name: Provide a unique name for each unique space conditioning system type in the building. If the same space conditioning system type is used in more than one location in the building, it is sufficient to list the unique space conditioning system type only once.  In order for one space conditioning system type to be considered the same as another, it must have the same description in Columns 2 through 9.

- Heating System Type: Indicate heating system type as furnace, central heat pump, boiler, hydronic, wood heat, wall furnace, room heat pump, or electric resistance if it meets the exception. An exception to Section 150.1(c)6 allows electric resistance heating only when it is supplemental to another system, as indicated by a capacity of < 2 kW or 7,000 Btu/hr, and a time-limiting control device that allows it to be operated for 30-minutes at a time.

- Heating Efficiency Type: AFUE, HSPF, COP

- Heating Efficiency Value: For central gas heating systems, the minimum efficiency required by the appliance efficiency standards is 81% AFUE. Heat pumps have an HSPF of 8.0 or higher. Other appliance types will have different efficiency levels (e.g., a gas wall furnace may have a minimum requirement of 73% AFUE or lower, depending on the size and type). Any gas heating appliance (or heat pump) sold in California is acceptable. The only electric heating appliance allowed is a heat pump.

- Cooling System Type: Indicate cooling system type or specify “no cooling system installed.” Categories include central air split system, central air package system, heat pump, room air or room heat pump, mini-split heat pump, or no cooling.

- Cooling Efficiency SEER: For central cooling systems, the minimum efficiency required by the appliance efficiency standards is 14 SEER.

- Cooling Efficiency EER: Other appliance types will have different efficiency levels (e.g., a room air conditioner may have a minimum requirement of 9 EER (when an appliance standard is an EER this is considered equivalent to a SEER). Any cooling appliance sold in California is acceptable.

- Distribution System Type: This could be ducted, radiant floor, piping, or ductless.

- Duct Location: If the system has ducts, indicate where they will be installed. Locations include attic, garage, conditioned space, radiant floor.

- Duct R-value: This value is from Package A. If system is ductless this field will be N/A.

- Thermostat Type: Select a setback thermostat or an Energy Management System (EMS) for most systems, or N/A if exempt. Controls for most systems can be by a device that allows a person to program up to 4 temperature setpoints within 24 hours.

- Comments: Include any comments here.

K. Ventilation Cooling

In climate zones 8-14, a whole house fan is required to provide ventilation. The requirement is found in Section 150.1(c)12.

- Required Whole House Fan Airflow Rate (CFM): 1.5 CFM per ft2 of conditioned floor area (auto complete).

- Minimum Attic Free Vent Area (in2): Minimum attic vent free area = Column 1 multiplied by 144 and divided by 750, which is equivalent to multiplying by 0.192 (auto complete).

L. Water Heating Systems

- Water Heating System Identification or Name: Provide a unique name for each unique water heating system type in the building. If the same water heating system type is used in more than one location in the building, it is sufficient to list the unique water heating system type only once.  In order for one water heating system type to be considered the same as another, it must have the same description in Columns 2 through 12.

- Water Heating System Type: Domestic Hot Water (DHW), Hydronic, Combined Hydronic, or Central. DHW is for domestic hot water, hydronic is a water heating system used for space heating only; combined hydronic are when the water heater will provide both space conditioning and domestic hot water. A central water heater serves multiple dwelling units in a multi-family building.

- Water Heater Type: Prescriptive Standards allow four options under Section 150.1(c)8.

Single Dwelling Unit

  - A single gas or propane instantaneous water heater with an input of 200,000 Btu per hour or less and no storage tank, and that meets the requirements of Sections 110.1 and 110.3.

  - A single gas or propane storage type water heater with an input of 105,000 Btu per hour or less, rated volume less than or equal to 55 gallons and that meets the requirements of Sections 110.1 and 110.3. The dwelling unit shall meet all of the requirements for Quality Insulation Installation (QII) as specified in the Reference Appendix RA3.5.

  - A single gas or propane storage type water heater with an input of 105,000 Btu per hour or less, rate volume of more than 55 gallons, and that meets the requirements of Sections 110.1 and 110.3.

Central System

  - All water heaters installed must comply with Sections 110.1 and 110.3. The distribution system shall be equipped with a demand recirculation control allowing pump operation to be based on measurement of hot water demand and hot water return temperature. The system shall have at least two loops. Buildings with 8 or less units do not have to comply with the demand recirculation requirement.

- Number of Water Heaters in System: In single family and multi-family with water heaters in each dwelling units the value is 1. For multi-family central systems serving multiple dwelling units enter the total number of water heaters.

- Water Heater Storage Volume (gal): Tank capacity in gallons. For instantaneous water heaters, enter N/A. For multi-family central systems enter the total storage volume.

- Fuel Type: Gas, Propane.

- Rated Input Type: Enter the equipment input rating type, for gas or propane fired system the units are Btuh, for electric fired system the units are kW.

- Rated Input Value: Enter the numeric value of rated input.

- Heating Efficiency Type: Energy Factor, AFUE, UEF or Thermal Efficiency. From product literature or a California Energy Commission directory.

- Heating Efficiency Value: Enter the value from product literature or a California Energy Commission directory

- Standby Loss (%): Applies only to large storage water heaters. Enter N/A for small storage or instantaneous water heaters.

- Exterior Insulation R-Value: Enter the R-value if exterior insulation on the storage tank is installed

- Back-Up Solar Savings Fraction: If compliance requires a back-up solar system, indicate the solar contribution (e.g., 0.30).  External calculations are required.

- Central DHW System Distribution Type: For multi-family buildings using a central distribution system a demand recirculation system with at least two distribution loops must be installed. This requirement applies to any building with 8 or more units. If the system is non-central or project is individual units enter N/A.

- Dwelling Unit DHW System Distribution Type: For a Central DHW this field shall be Standard. If non-central then pick from Standard, Demand Recirculation – Manual Control, Demand Recirculation – Sensor Control.

M. Space Conditioning Systems and Water Heating Systems in Multifamily Dwelling Units

- Dwelling Unit Name: Enter one unique name for each of the number of dwelling units identified in Section A field 06.

- Dwelling Unit Total CFA: Enter the conditioned floor area for the dwelling unit.

- Central Water Heating System Identification or Name: Select one of the central DHW system names

- Dwelling Unit Water Heating System Identification or Name: Select one of the Dwelling Unit water heating system names entered in Section L. If more than one water heating system type is needed in the dwelling unit, enter another row of data for the dwelling unit and select the additional water heating system name.

- Dwelling Unit Space Conditioning System Identification or Name: Select one of the Space conditioning system names that were entered in Section J. If more than one space conditioning system type is needed in the dwelling unit, enter another row of data for the dwelling unit and select the additional space conditioning system name

- Comments: Include any comments here.

N. HERS Verification Summary

- Duct Leakage verification: All duct systems must meet maximum duct leakage requirements. Typically the maximum leakage is 5% but varies for when the duct leakage test is performed and the type of building (single family, townhouse, multifamily). The only exception is if the heating and cooling systems are ductless.

- Zonally Controlled Systems - Bypass Dampers: The prescriptive requirements preclude the use of bypass ducts in association with zonally controlled systems. A HERS Rater will verify that zonally controlled systems have no bypass ducts.

- Refrigerant Charge Verification: Some type of refrigerant charge verification or Fault Indicator Display is required in climate zones 2 and 8-15 for most common systems such as ducted split and packaged systems, and mini-split systems. See Section 150.1(c)7A. or Reference Residential Appendix RA3.2. If a building is built in climate zones 1, 3-17 or 16, or has no cooling system, no refrigerant charge verification is required.

- Central System Air Handlers - Airflow Rate and Fan Efficacy Verification: Unless a building has no cooling system or has a non-ducted cooling system, the system must meet mandatory and prescriptive requirements for an airflow greater than 350 CFM per ton of nominal cooling capacity, and a fan efficacy less than or equal to 0.58 W/CFM. See 150.0(m)13, 150.1(c)10, and Reference Residential Appendix RA3.

- Indoor Air Quality Mechanical Ventilation: All new dwellings are required to meet the whole-building mechanical ventilation airflow rate according to ASHRAE 62.2 is required (RA3.7).

Documentation Declaration Statements

- The person who prepared the CF1R will sign and complete the fields for their name, company (if applicable), address, phone number, certification information (if applicable), date and signature (may be electronic).

- The person who is assuming responsibility for the project being built to comply with Title 24, Part 6, will complete the fields for their name, company (if applicable), address, phone number, license number (if applicable), date and signature (may be electronic).

Registration

The CF1R must be registered with a HERS provider prior to submitting for a building permit.

Standards References

- Thermostats

  - Thermostat requirements are found in Section 110.2(c) with special requirements for heat pumps in Section 110.2(b). Controls for most systems can be by a central energy management control system (“EMS”) or a setback thermostat with a mechanism allowing a person to program up to 4 temperature setpoints within 24 hours (“setback”).

EXCEPTIONS: 	If the heating system type is a gravity gas wall, floor or room heater, non-central electric heater, fireplace or decorative gas appliance, or wood stove, a setback thermostat or energy management control system is not required.

If the cooling system type is a room air conditioner or room air conditioner heat pump setback thermostat or energy management control system is not required.

- Water Heaters:

Section 150.1(c) allows a limited number of conditions for water heating. If conditions other than these are proposed, the prescriptive compliance approach cannot be used:

Single Dwelling Unit

  - A single gas or propane instantaneous water heater with an input of 200,000 Btu per hour or less and no storage tank, and that meets the requirements of Sections 110.1 and 110.3.

  - A single gas or propane storage type water heater with an input of 105,000 Btu per hour or less, rated volume less than or equal to 55 gallons and that meets the requirements of Sections 110.1 and 110.3. The dwelling unit shall meet all of the requirements for Quality Insulation Installation (QII) as specified in the Reference Appendix RA3.5.

  - A single gas or propane storage type water heater with an input of 105,000 Btu per hour or less, rate volume of more than 55 gallons, and that meets the requirements of Sections 110.1 and 110.3.

Central System

  - All water heaters installed must comply with Sections 110.1 and 110.3. The distribution system shall be equipped with a demand recirculation control allowing pump operation to be based on measurement of hot water demand and hot water return temperature. The system shall have at least two loops. Buildings with 8 or less units do not have to comply with the demand recirculation requirement.

| A. General Information |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 01 | Project Name: | <<User Input: Text>> | 02 | Date Prepared: | <<User Input: Date>> |
| 03 | Project Location: | <<User Input: String>> | 04 | Building Front Orientation (deg or cardinal): | <<User Input: IntegerNonnegative>> |
| 05 | CA City: | <<User Input: String>> | 06 | Number of Dwelling Units: | <<User Input: IntegerNonnegative; note if A11=Single Family, then the value=1; else if A11=Multifamily, then the value ≥2>> |
| 07 | Zip Code: | <<User Input: Zipcode>> | 08 | Fuel Type: | <<User selects from list: Natural Gas, Propane>> |
| 09 | Climate Zone: | <<User selects from list: 1, 2, 3, 4, 5, 6, 7, 8, 9, 10, 11, 12, 13, 14, 15, 16>> | 10 | Total Conditioned Floor Area (ft2): | <<User Input: IntergerNonnegative>> |
| 11 | Building Type: | <<user select one from list: *Single Family *Multi Family>> | 12 | Slab Area (ft2): | <<User Input: IntegerNonnegative>> |
| 13 | Project Scope: | << user select as many as are applicable from list: *Newly Constructed Building *New Addition greater than 1,000 ft2>> | 14 | Exceptions to Fenestration U-factor and SHGC 150.1(c)3A | <<User selects from list: *NA (do not allow other entries) *Installing ≤ 3ft2 Glass in Door *Installing ≤ 3ft2 Tubular Skylight *Installing ≤ 16ft2 Skylight |

| B. Opaque Surface Details – Framed (Section 150.1(c)1) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |  | 07 | 08 | 09 | 10 | 11 |
| Tag/ID | Assembly Type | Frame Type | Frame Depth (inches) | Frame Spacing (inches) | Proposed |  |  |  |  | Required | Comments |
|  |  |  |  |  | Cavity R-value | Continuous Insulation R-value | U-Factor | Appendix JA4 Reference |  | U-Factor from Package A |  |
|  |  |  |  |  |  |  |  | Table | Cell |  |  |
| <<User Input: ObjectNamePermissive>> | <<User selects from list: Ceiling, Floor, Roof, Wall>> | << User selects from list: Wood or Metal>> | << User selects from list: 2x4, 2x6, 2x8, 2x10, 2x12 or 2x14>> | << user selects from list: 16 inches on center,24 inches on center, 48 inches on center>> | <<User Input: DecimalNonnegative>> | <<User Input: DecimalNonnegative>> | <<User Input: DecimalNonnegative>> | <<User Input: JA_TableID>> | <<User Input: JA_TableCell>> | <<User Input: DecimalNonnegative>> | <<User Input: Text>> |

| C. Opaque Surface Details – Non-framed (Section 150.1(c)1) |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 | 07 | 08 | 09 | 10 | 11 |
| Tag/ID | Assembly Type | Assembly Materials | Thickness (inches) | Proposed |  |  |  |  | Required | Comments |
|  |  |  |  | Core Insulation R-value | Continuous Insulation R-value | U-Factor | Appendix JA4 Reference |  | U-Factor from Package A |  |
|  |  |  |  |  |  |  | Table | Cell |  |  |
| <<User Input: ObjectNamePermissive>> | <<User selects from list: Roof, Wall>> | <<User selects from list: SIP OSB, SIP 2x, SIP 4x, SIP I-Joist>> | <<User Input: DecimalNonnegative>> | <<User Input: DecimalNonnegative>> | <<User Input: DecimalNonnegative>> | <<User Input: DecimalNonnegative>> | <<User Input: JA_TableID>> | <<User Input: JA_TableCell>> | <<User Input: DecimalNonnegative>> | <<User Input: Text>> |

| D. Opaque Surface Details – Mass Walls (Section 150.1(c)1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 | 07 |  |  | 08 |  |  |  | 09 |  | 10 | 11 |  |  | 12 |  |
| Tag/ID | Walls Above Grade | Mass Type | Mass Thickness (inches) | Interior Furring Strip Thickness (inches) | Exterior Furring Strip Thickness (inches) | Proposed |  |  |  |  |  |  |  |  |  | Required |  |  |  |  |
|  |  |  |  |  |  | Interior Insulation |  |  | Exterior Insulation |  |  |  | Appendix JA4 Reference |  |  | Interior Insulation |  |  | Exterior Insulation |  |
|  |  |  |  |  |  | R-value | U-factor |  | R-value |  | U-factor |  | Table |  | Cell | R-value |  | U-factor | R-value | U-factor |
| <<User Input: ObjectNamePermissive>> | <<User selects from list: Yes or No>> | <<User selects from list: Clay Brick, Clay Hollow Unit, CMU Light Weight, CMU Medium Weight, CMU Normal Weight, Concrete or Insulating Concrete Forms>> | <<User Input: DecimalNonnegative>> | <<User Input: DecimalNonnegative>> | <<User input: DecimalNonnegative>> | <<User Input: DecimalNonnegative; Note: Either R-value or U-factor is required, not both. The same descripted should be used throughout this table >> | <<User Input: DecimalNonnegative; Note: Either R-value or U-factor is required, not both. The same descripted should be used throughout this table >> |  | <<User Input: DecimalNonnegative; Note: Either R-value or U-factor is required, not both. The same descripted should be used throughout this table >> |  | <<User Input: DecimalNonnegative; Note: Either R-value or U-factor is required, not both. The same descripted should be used throughout this table >> |  | <<User Input: JA_TableID>> |  | <<User Input: JA_TableCell>> | <<User Input: DecimalNonnegative; Note: Either R-value or U-factor is required, not both. The same descripted should be used throughout this table >> |  | <<User Input: DecimalNonnegative; Note: Either R-value or U-factor is required, not both. The same descripted should be used throughout this table >> | <<User Input: DecimalNonnegative; Note: Either R-value or U-factor is required, not both. The same descripted should be used throughout this table >> | <<User Input: DecimalNonnegative; Note: Either R-value or U-factor is required, not both. The same descripted should be used throughout this table >> |

| E. Slab Insulation (Table 150.1-A) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 |  | 03 |  | 04 |
| Floor Type | Proposed |  | Required |  | Comments |
|  | Insulation R-value | Insulation U-factor | Insulation R-value | Insulation U-factor |  |
| <<User selects from list: Slab on grade, Slab raised>> | <<User Input: DecimalNonnegative>> | <<User Input: DecimalNonnegative>> | <<User Input: DecimalNonnegative>> | <<User Input: DecimalNonnegative>> | <<User Input: Text>> |
| - Heated slab floors require mandatory slab insulation (see Table 110.8-A). |  |  |  |  |  |

F. Ceiling/Roof Insulation (Section 150.1(c)1A)

| 01 | 02 | 03 | 04 | 05 | 06 | 07 |
| --- | --- | --- | --- | --- | --- | --- |
| Option | Air Space (Yes/No) | Above Roof Deck R-value | Below Roof Deck R-value | Ceiling Insul. R-value | Radiant Barrier Required? | Vented Attic? (Yes/No) |
| <<user selects from list: *Option A; or *Option B; or *Option C>> | <<user selects from list: *Yes; or *No>> | <<if F01 contains Option A, A09 = 4,8-16, and F02 = No; then value equals R-8; else if F02 = Yes, then value equals R-6; Else NA>> | << if F01 contains Option B, A09 = 4,8-16, and F02 = No, then value equals R-18; else if F02 = Yes, then value equals R-13; Else NA>> | <<if F01 contains Option A or Option B, and A09 = 3,5-7, then value equals R-30; If F01 contains Option C and A09 = 2-10, then value equals R-30; Else value equals R-38>> | <<if F01 contains Option A or Option C, and A09 = 2-15, then value equals ‘Required’; If F01 contains Option B and A09 = 2,3,5-7, then value equals ‘Required’; Else value equals NA>> | <<user selects from list: *Yes; or *No; If No is selected, display ‘Do Not Proceed’>> |
| Note: - Where insulation is installed above the roofing membrane or above the layer used to seal the roof from water penetration the insulation shall have a maximum water absorption of 0.3 percent by volume when tested according to ASTM Standard C272. |  |  |  |  |  |  |

| G. Roofing Products (Cool Roof) (Section 150.1(c)11) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 | 07 | 08 | 09 | 10 | 11 | 12 |  | 13 |
| Tag/ID | Mass Roof 25 lb/ft2 or greater | Roof Pitch | Method of compliance | Product Type | CRRC Product ID Number | Proposed |  |  |  | Minimum Required |  |  |  |
|  |  |  |  |  |  | Initial Solar Reflectance | Aged Solar Reflectance | Thermal Emittance | SRI (Optional) | Aged Solar Reflectance | Thermal Emittance | SRI (Optional) |  |
| <<User Input: ObjectNamePermissive>> | <<User selects from list: Yes or No; If Yes, meets cool roofs done. Else No go to 03>> | <<User selects from list:Roof pitch is > 2:12 or Roof pitch is ≤ 2:12>> | << User selects from list: Not in an applicable climate zone, Aged Solar Reflectance and Thermal Emittance, or SRI>> | << User selects from CRRC product list: Asphalt Shingles, Built-up Roofing, Clay Roof Tiles, Factory Applied Coating, Field Applied Coating, Metal Roof, Modified Bitumin Sheet, Roof Pavers, Single Ply Thermoplastic, Single Ply Thermoset, Metal Shakes Shingles Fluid Applied Membrane, Polymer Composite Steep Slope, Spray Polyurethane Foam, Stone Aggregate Ballast>> | <<User input From the CRRC Directory if user knows what they are going to install. Else NA>> | <<if cell 04 = ‘Not in an applicable climate zone’ then result = NA; Else If the user knows what they are installing, the user enters the DecimalNonnegative value from the CRRC Directory, Else allow user to enter NA>> | <<if cell 04 = ‘Not in an applicable climate zone’ then result = NA; Else User Input: DecimalNonnegative>> | <<if cell 04 = ‘Not in an applicable climate zone’ then result = NA; Else User Input: DecimalNonnegative>> | <<if CoolRoofComplianceMethod does not equal SolarReflectanceIndex then result is NA; Else user enters value from CRRC Directory or from a completed SRI Worksheet>> | << for Climate Zones 13 & 15 shown in Section A cell 09, and if cell 03 of Section G indicates a ≤ 2:12 roof pitch place a 0.63 else for Climate Zones 10-15, and if G03 indicates roof pitch is >2:12 place a 0.20. Else for all other combinations of climate zone and roof pitch, the result = NA>> | <<for Climate Zones 13 & 15 shown in Section A cell 09, and if cell 03 of Section G indicates ≤ 2:12 roof pitch and G04 is SRI than thermal emittance is 0.85>> if cell 04 is Aged Solar Reflectance and Thermal Emittance than Thermal Emittance is 0.75. For Climate Zones 10-15 if G03 indicates roof pitch is > 2:12 then Thermal emittance is 0.75. Else for all other combination of climate zone, roof pitch and cool roof compliance method, the result = NA>> | << if cell 04 indicates SRI and cell 03 indicates ≤ 2:12 place 75 for climate 13 and 15 based on Section A cell 09. But if cell 03 indicates >2:12, SRI should be 16 for climate zones 10-15 based on Section A cell 09. All other climate zone and roof pitch shall be NA.>> |  |
| Notes: - Any roof area covered by building integrated photovoltaic panels and solar thermal panels is exempt from the above Cool Roof requirements. - Liquid field applied coatings must comply with installation criteria from section 110.8(i)4. |  |  |  |  |  |  |  |  |  |  |  |  |  |

| H. Fenestration/Glazing Allowed Areas and Efficiencies |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 | 07 |
| Maximum Allowed Fenestration Area for All Orientations (ft2) | Maximum Allowed West-Facing Fenestration Area Only (ft2) | Maximum Allowed U-factor (Windows) | Maximum Allowed U-factor (Skylights) | Maximum Allowed SHGC (Windows) | Maximum Allowed SHGC (Skylights) | Comments |
| <<Calculated (CFA x 0.2) for all orientation>> | << If A09 equals 1, 3 or 5 then report N/A; Else Calculated (CFA X 0.05) for west-facing fenestration>> | <<0.32>> | <<If A14 contains Installing ≤ 3ft2 Tubular Skylight, or Installing ≤ 16ft2 Skylight, then report 0.55; Else report 0.32>> | <<If A09 equals 1, 3 or 5 then report N/A; Else report 0.25>> | <<If A14 contains Installing ≤ 3ft2 Tubular Skylight, or Installing ≤ 16ft2 Skylight, then report 0.30; Else report 0.25>> | <<User Input: Text>> |

| I. Fenestration Proposed Areas and Efficiencies Note: If meeting Exception 1 to 150.1(c)3A, Installing ≤ 3ft2 glass in door, it is assumed to meet the minimum required U-factor (0.32) & SHGC (0.25). If meeting Exception 1 to 150.1(c)3A, Installing ≤ 3ft2 tubular skylight, it is assumed to meet the minimum required U-factor (0.55) & SHGC (0.30). |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 |  | 04 | 05 | 06 | 07 | 08 | 09 | 10 | 11 | 12 | 13 | 14 |
| Tag/ID | Fenestration Type | Frame Type |  | Dynamic Glazing | Orientation N, S, W, E | Number of Panes | Proposed Fenestration Area (ft2) | Proposed West Facing Fenestration Area (ft2) | Proposed U-factor | Source | Proposed SHGC | Source | Exterior Shading Device | Combined SHGC from CF1R-ENV-03 |
| <<User Input: String (max 50 characters)>> | <<User selects from list: *fixed window *operable window *skylight *tubular skylight (only allow if A14 contains Installing ≤ 3ft2 tubular skylight) *glass in door (only allow if A14 contains Installing ≤ 3ft2 glass in door) | <<User selects from list: Metal framing, Metal framing with thermal break, Nonmetal framing >> |  | <<User selects from list: None, Chromogenic glazing, Integrated shading device>> | <<User selects from list: North, East, South, West>> | <<User selects from list: Double pane, Single pane, Triple pane>> | <<If I05 equals North, East or South, then enter user defined number in this column>> | <<If I05 equals West, then enter user defined number in this column>> | <<User Input: DecimalNonnegative >> | <<User selects from list: NFRC,Tables 110.6-A and 110.6-B, Equations NA6-1 And NA6-2, or Area-weighted Average (ENV-02)>> | <<User Input: DecimalNonnegative >> | <<User selects from list: NFRC,Tables 110.6-A and 110.6-B, Equations NA6-1 And NA6-2, or Area-weighted Average (ENV-02)>> | <<User selects from list: Drop Arm Awnings, Louvered Sun Screen, Low Sun Angle Louvered Sun Screen, None, Operable Awnings, Overhang, Retractable Awnings, Roll Down Blinds Or Slats, Standard Bug Screen, Sun Screen, Vertical Roller Or Shades>> | <<If I12 equal to None, or I04 contains Chromogenic glazing, then report N/A; Else report value from CF1R-ENV-03>> |
| 15 | Total Proposed Fenestration Area |  |  |  |  |  |  |  |  |  |  |  | <<calculate (sum of column I07 and I08)>> |  |
| 16 | Maximum Allowed Fenestration Area |  |  |  |  |  |  |  |  |  |  |  | <<Report Value from H01>> |  |
| 17 | Compliance Statement |  | <<If I15 is equal to or less than I16 then report: Design Complies with the Total Allowed Fenestration Area Else report: Total Proposed Fenestration Area Exceeds Allowable, Do Not Proceed>> |  |  |  |  |  |  |  |  |  |  |  |
| 18 | Total Propose West-Facing Fenestration Area |  |  |  |  |  |  |  |  |  |  |  | <<calculate (sum of column I08)>> |  |
| 19 | Maximum Allowed West Fenestration Area |  |  |  |  |  |  |  |  |  |  |  | << Report Value from H02>> |  |
| 20 | Compliance Statement |  | <<If I18 is equal to or less than I19, or I19 equals N/A, then report: Design Complies with the Total Allowed West-facing Fenestration Area Else report: Total Proposed West-facing Fenestration Area Exceeds Allowable, Do Not Proceed>> |  |  |  |  |  |  |  |  |  |  |  |
| 21 | Proposed Fenestration U-factor (Windows) |  |  |  |  |  |  |  |  |  |  |  | <<If I02 contains fixed or operable window, and if all associated values listed in column I09 are less than or equal to 0.32, then enter the largest single value from list; Else enter the weighted average value from the CF1R-ENV-02>> |  |
| 22 | Required Fenestration U-factor (Windows) |  |  |  |  |  |  |  |  |  |  |  | <<Report Value from H03>> |  |
| 23 | Compliance Statement |  | >>If I21 is equal to or less than I22 then report: Design Complies with the Maximum Allowed Fenestration U-value Else report: Fenestration U-value Exceeds Allowable, Do Not Proceed |  |  |  |  |  |  |  |  |  |  |  |
| 24 | Proposed Fenestration SHGC (Windows) |  |  |  |  |  |  |  |  |  |  |  | <<If I02 contains fixed or operable window, and if all associated values listed in column I11 and I14 are less than or equal to 0.25 then enter the single largest value from the two lists; Else enter the weighted average value from the CF1R-ENV-02>> |  |
| 25 | Required Fenestration SHGC (Windows) |  |  |  |  |  |  |  |  |  |  |  | <<Report Value from H05>> |  |
| 26 | Compliance Statement |  | <<If I24 is equal to or less than I25, or I25 equals N/A, then report: Design Complies with the Maximum Allowed Fenestration SHGC Else report: Fenestration SHGC Exceeds Allowable, Do Not Proceed>> |  |  |  |  |  |  |  |  |  |  |  |
| 27 | Proposed Fenestration U-factor (Skylights) |  |  |  |  |  |  |  |  |  |  |  | <<If I02 contains Skylight then enter the single largest associated value from I09; If I02 does not contain Skylight then value equals NA>> |  |
| 28 | Required Fenestration U-factor (Skylights) |  |  |  |  |  |  |  |  |  |  |  | <<Report Value from H04>> |  |
| 29 | Compliance Statement |  | <<If I27 equals NA then value equals NA; If I27 is equal to or less than I28 then report: Design Complies with the Maximum Allowed Fenestration U-value Else report: Fenestration U-value Exceeds Allowable, Do Not Proceed |  |  |  |  |  |  |  |  |  |  |  |
| 30 | Proposed Fenestration SHGC (Skylights) |  |  |  |  |  |  |  |  |  |  |  | <<If I02 contains Skylight then enter the single largest associated value from columns I11 or I14; If I02 does not contain Skylight then value equals NA>> |  |
| 31 | Required Fenestration SHGC (Skylights) |  |  |  |  |  |  |  |  |  |  |  | <<Report Value from H06>> |  |
| 32 | Compliance Statement |  | <<If I30 equals NA then value equals NA; If I30 is equal to or less than I31 then report: Design Complies with the Maximum Allowed Fenestration SHGC Else report: Fenestration SHGC Exceeds Allowable, Do Not Proceed>> |  |  |  |  |  |  |  |  |  |  |  |

| J. Space Conditioning (SC) Systems – Heating/Cooling/Ducts (Section 150.1(c)7) <<for single family dwellings, the data registry shall require one CF2R-MCH-01c; else for Multifamily Dwellings, the Data Registry shall require one CF2R-MCH-01c for each dwelling unit in the building as identified in Section M>> |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 | 07 | 08 | 09 | 10 | 11 | 12 |
| SC System Identification or Name | Heating System Type | Heating Efficiency Type | Heating Efficiency Value | Cooling System Type | Cooling Efficiency SEER | Cooling Efficiency EER | Distribution System Type | Duct Location | Duct R-value | Thermostat Type | Comments |
| <<User Input: ObjectNamePermissive>> | <<User selects from list: Boiler, Central gas furnace, Central large packaged HP, Central split HP, Combined hydronic, Combined hydronic forced air, Ductless HP, Electric, Gas space heater, Gas wall furnace, Hydronic, Hydronic forced air, Hydronic HP, Hydronic HP forced air, Room HP, Wood Heat, Packaged gas furnace>> | <<User selects from list: AFUE, COP, HSPF >> | <<User Input: DecimalNonnegative>> | <<User selects from list: Central large packaged AC, Central large packaged HP, Central packaged AC, Central packaged HP, Central split AC, Central split HP, Ductless AC, Ductless HP, Evaporative direct, Evaporative indirect, Evaporative indirect direct, Evaporatively cooled condenser, Gas absorption AC, Hydronic, Hydronic HP, Hydronic HP forced air, Ice storage AC, No cooling, Room AC, Room HP>> | <<User Input: Decimal1PlaceNonnegative>> | <<User Input: Decimal1PlaceNonnegative>> | <<User selects from list: Ducted system, Ductless, Piping, Radiant>> | <<User selects from list: Conditioned space-, No ducts, Ducts located in multiple places, Outdoor locations, Unconditioned attic, Unconditioned crawl space, Unconditioned garage>> | <<if system is ductless, then display NA; elseif F01 = ‘Option A’ or ‘Option B’ AND CZ = 1, 2, 4, or 8-16, then display R-8; else display R-6>> | <<User selects from list: Energy Management System (EMS), Setback thermostat, or NA>> | <<User Input: Text>> |
| - Central gas furnaces have a minimum efficiency of 81% AFUE, heat pumps 8.0 HSPF. Any gas heating appliance sold in California will meet the minimum appliance efficiency standard and is allowed. Heat pumps and mini-split heat pumps are the only type of electric heating system allowed. - Central cooling systems and heat pumps have a minimum efficiency of 14 SEER. Any cooling appliance sold in California will meet the minimum appliance efficiency standard and is allowed. - The prescriptive requirements preclude the use of bypass ducts in association with zonally controlled systems. A HERS Rater shall verify that zonally controlled systems have no bypass ducts. |  |  |  |  |  |  |  |  |  |  |  |

| K. Ventilation Cooling in Climate Zones 8-14 (Section 150.1(c)12) |  |
| --- | --- |
| 01 | 02 |
| Required Airflow Rate (CFM) (1.5 CFM per ft2 of Conditioned Floor Area) | Minimum Attic Vent Free Area (in2) (Required Airflow Rate x 0.192) |
| <<[(1.5 CFM per ft2) x A10]>> | <<[(column K01) x 0.192]>> |

| L. Water Heating Systems (Section 150.1(c)8) List water heaters and boilers for both domestic hot water (DHW) heaters and hydronic space heating. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 | 07 | 08 | 09 | 10 | 11 | 12 | 13 | 14 | 15 |
| Water Heating System ID or Name | Water Heating System Type | Water Heater Type | # of Water Heaters in system | Water Heater Storage Volume (gal) | Fuel Type | Rated Input Type | Rated Input Value | Heating Efficiency Type | Heating Efficiency Value | Standby Loss (%) | Exterior Insul. R-Value | Back-Up Solar Savings Fraction | Central DHW System Distribution Type | Dwelling Unit DHW System Distribution Type |
| <<user input text; do not allow duplicate entries in this column; allow user to enter multiple rows as needed>> | <<user pick from list, DHW, Central, Hydronic, Combined Hydronic>> | <<if L02 = DHW, then user pick one from list: Small Storage, Large Storage, Consumer Storage, Commercial Storage, Consumer Instantaneous, Small Instantaneous, or Residential-Duty Commercial Storage; Else user pick one from list: Large Storage, Small Storage, Boiler, Large Instantaneous, Small Instantaneous, Consumer Instantaneous, Commercial Instantaneous, Consumer Storage, Commercial Storage, Residential-Duty Commercial Storage or Indirect>> | <<user input: IntergerNonnegative>> | <<user enter DecimalNonnegative xxx.x, or if A03 = instantaneous enter N/A>> | <<user pick one from list: Natural Gas, Propane>> | <<If Fuel Type L06 = Natural Gas, Propane then Rated Input Type = Btu/Hr. >> | <<user input: IntegerNonnegative; range check, if L03 = Small storage, Large storage, Consumer Storage, Commercial Storage, or Residential-duty commercial storage, then value must be ≤ 105,000>> | << user pick from list: *Energy Factor, *AFUE, *Thermal Efficiency*Uniform Energy Factor>> | <<user input: DecimalNonnegative>> | <<user input: DecimalNonnegative x.xx, or select N/A>> | <<user input: DecimalNonnegative xx, or select N/A>> | <<user input: DecimalNonnegative, or select N/A>> | <<user pick from list: *N/A - not a central DHW system *Multi-family: Recirculating demand control>> | <<If L14≠ N/A, then value= *Standard Distribution System Elseif L14 = N/A, and Water Heat Type = Small storage, Large storage, Consumer Storage, Commercial storage, or Residential-duty commercial storage, then user pick form list: *HERS-Verified Pipe Insulation; or *HERS-Verified Compact Hot Water Distribution System; Else user pick from list: *Standard Distribution System; *Demand Recirculation – Manual Control *Demand Recirculation – Sensor Control *Point of Use>> |

| M. Space Conditioning Systems and Water Heating Systems in Multifamily Dwelling Units << If Section A field 11 = Single Family, then display the section does not apply message; else require a minimum of one (1) row of data to be entered in this section for each of the quantity of dwelling units entered in A06; require one (1) unique dwelling unit name in M01 for each of the quantity of dwelling units entered in A06; >> <<the data registry shall require one CF-2R-MCH-01c for each of the dwelling units reported in this section>> <<the data registry shall require one CF2R-PLB-01 for buildings that have one (1) or more systems reported in L14 that do not have N/A as value>> <<The data registry shall require one (1) CF2R-PLB-02 for each of the dwelling unit names reported in M01>> |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| 01 | 02 | 03 | 04 | 05 | 06 |
| Dwelling Unit Name | Dwelling Unit Total CFA (ft2) | Central Water Heating System Identification or Name | Dwelling Unit Water Heating System Identification or Name | Dwelling Unit Space Conditioning System Identification or Name | Comments |
| <<user entry text; require at least the same quantity of unique dwelling unit names to be entered in this column as are identified in A06; do not allow duplicate dwelling unit names>> | <<user enter integer ≥ 0; for each dwelling unit name in M01>> | << if all values in column L14=N/A, then value = N/A; else, user pick from list comprised of all the Water Heating Systems in L01 that do not have a value of N/A in L14, or allow user to pick N/A if the dwelling unit is not served by a central DHW system; | <<user pick multiple items from list comprised of all the Water Heating System names in L01 that have a value in L14=N/A>> | <<user pick multiple items from list comprised of all the SC System names in J01>> | <<user input text>> |

N. HERS Verification Summary The enforcement agency shall pay special attention to the HERS Measures specified in this checklist below. A registered Certificate of Verification for all the measures specified shall be submitted to the building inspector before final inspection.

Duct Leakage Verification- Section 150.0(m)11 - Duct leakage testing is required (Residential Appendix RA3.1) in all climate zones for ducted heating and cooling systems. - System is zonally controlled. No bypass ducts are allowed, as confirmed by HERS verification

Zonally Controlled Systems – Bypass Dampers - Section 150.1(c)13 - If system is zonally controlled, no bypass ducts are allowed, as confirmed by HERS verification (See RCM Appendix F)

Refrigerant Charge Verification – Section 150.1(c)7a - Refrigerant Charge Testing is required (Residential Appendix RA3.2) in climate zones 2 and 8-15 for all air source A/C and heat pumps. - Some exceptions apply to factory charged package systems

Central System Air Handlers – Air Flow and Fan Efficacy Verification - Section 150.0(m)13 - Airflow (min 350 cfm/ton) and Fan Efficacy (max 0.58 watts/cfm) on systems with ducted air conditioning will be field verified by a HERS rater or Return Duct and Filter System Design according to tables 150.0-C/D will be HERS verified - Heat-only systems with Central Fan Integrated (CFI) ventilation are required to have less than 0.58 watts per cfm as verified by a HERS rater.

Indoor Air Quality Mechanical Ventilation - Mechanical ventilation airflow rate according to ASHRAE 62.2 is required to be verified by a HERS rater (RA3.7)

<<if L03 = Small Storage, Large storage, Consumer Storage or Commercial Storage, and L05 ≤ 55 gallons, then result = true and display text; Else result = false and do not display text>> Quality Insulation Installation - Section 150.1(c)8ii - QII is required when installing a single gas or propane water heater with an input of 105,000 Btu per hour or less, rated volume less than or equal to 55 gallons and that meets the requirements of Sections 110.1 and 110.3. - The dwelling unit shall meet all of the requirements of Quality Insulation Installation (QII) as specified in Reference Appendix RA3.5 and be verified by a HERS rater.